姓名：            张洋                             学号：             20232411                   d

《卫星导航定位基础》课后作业（一）

利用广播星历计算BDS卫星的位置

任务部分：每位同学登陆网站（ftp://epncb.oma.be/pub/obs/BRDC/2025/或ftp://pub:tarc@ftp2.csno-tarc.cn/almanac/2025）下载一个不同天的广播星历文件，批量计算一天所有BDS卫星的位置、高度角和方位角，并绘制二维和三维分布图，并与精密星历进行对比评价轨道精度。

- 算法

北斗广播星历由 6 个开普勒轨道根数、3 个摄动线性改正项和 6 个周期改正项系数 等一共 15 个描述轨道特征的参数和 1 个星历参考时间组成，其广播星历参数的更新时间是 1h。

首先，对2月24日广播星历进行预处理，数据预处理步骤如下：

提取北斗卫星数据：读取广播星历文件，找到数据块起始位置，筛选出以'C'开头的北斗卫星数据行，按 8 行一组收集，存储到新文件all_C.txt。如下图所示：

解析数据并存为字典：读取all_C.txt，确定数据起始行，计算数据块个数。逐组解析数据，将每行特定位置数据对应卫星的不同参数，存入字典，所有字典构成列表。最后将列表数据写入北斗卫星.csv文件。如下图所示：

接着，读取上述csv文件中的2月24日的星历参数，进行北斗卫星坐标位置的计算。如下图表示：

t是信号发射时刻的北斗时。tk 是信号发射时刻t 和星历参考时间toe 之间的时间差，并考虑到跨过一周开始或结束的时间，其计算公式为：（时间差是由星历得到的信号传播时间差吗？星历是地面监控站接收到的卫星发出来我们接受到的星历）。

计算得出北斗卫星的坐标位置，输出为文件 北斗卫星位置.txt。根据该文件的数据绘制2D、3D图来可视化展示卫星位置。

然后，对2月24日精密星历数据进行预处理。筛选出北斗卫星的整小时的精密星历数据并按照卫星序号归类，输出为新文件 sp3_data.txt。如图：

继而，对sp3_data.txt 的2月24日数据真值和 北斗卫星位置.txt 的2月24日数据计算值进行误差计算，百分差计算：

误差：|真值 – 计算值|；

百分差：|真值 – 计算值| / 真值；

最后，计算卫星数据计算的平均误差和平均百分差，作为整个程序的精度指标，绘制误差散点图和箱线图对误差分布进行评估。

- 代码

运行环境：python 3.13.2；

库依赖：math, pandas, numpy, os, csv, matplotlib, plotly;

星历时间：2025年2月24日；

代码结构：

main.py – 解析广播星历文件，数据预处理程序；

draw2D.py – 绘制卫星位置的2D图像；

draw3D.py – 绘制卫星位置的3D图像；

sel6-45.py – 筛选出计算值中卫星编号为C06-C45的数据;

sp3.py – 解析精密星历文件，数据预处理程序；

ana.py – 计算误差与百分差；

config.py – 计算所有卫星的平均误差和平均百分差；

plot.py – 绘制计算误差的箱线图；

plot1.py – 绘制计算误差的分布散点图；

具体代码如下：

main.py:

import math as m

import numpy as np

import csv

import os

def rx(fai):

result = np.asmatrix([[1, 0, 0], [0, m.cos(fai), m.sin(fai)], [0, -1 * m.sin(fai), m.cos(fai)]])

return result

def rz(fai):

result = np.asmatrix([[m.cos(fai), m.sin(fai), 0], [-1 * m.sin(fai), m.cos(fai), 0], [0, 0, 1]])

return result

def reader_GNSS(filename):

infile = open("BRDC00GOP_R_20250550000_01D_MN.rnx", "r")

# 读取原星历文件

content = infile.readlines()  # 按行读取文件

infile.close()  # 关闭文件

start_num = 0

all_C = []  # 存储北斗星历数据块

for i in range(len(content)):  # len(content)为文件的行数

if content[i].find("                                                            END OF HEADER") != -1:

start_num = i + 1  # 找到数据块开始的位置

processed_lines = set()  # 用于记录已经处理过的行号

for i in range(len(content) - start_num):  # 遍历数据块

line = content[start_num + i]  # 第i行

if line[0] == 'C':

for j in range(0, 8):

line_index = start_num + i + j

if line_index < len(content) and line_index not in processed_lines:

all_C.append(content[line_index])

processed_lines.add(line_index)

outFile = open("all_C.txt", 'w')  # 存储为新的文件（只含有北斗卫星）

outFile.writelines(all_C)

outFile.close()

# 接下来是计算卫星位置的程序

with open('all_C.txt', 'r') as f:

if f == 0:print("不能打开文件！")

else:print("导航文件打开成功！")

nfile_lines = f.readlines()  # 按行读取N文件

print(len(nfile_lines)) # 打印N文件的行数 9216

f.close()

start_num = 0

for i in range(len(nfile_lines)): # 遍历N文件

if nfile_lines[i].find('END OF HEADER') != -1:

start_num = i + 1

n_dic_list = [] # 存储字典

n_data_lines_nums = int((len(nfile_lines) - start_num) / 8) # 计算数据块的个数

print("一共%d组数据" % (n_data_lines_nums))

# 第j组，第i行

for j in range(n_data_lines_nums):

n_dic = {}

for i in range(8):

data_content = nfile_lines[start_num + 8 * j + i] # 第j组，第i行

n_dic['数据组数'] = j + 1

if i == 0:

n_dic['卫星PRN号'] = str(data_content[0:3])

n_dic['历元'] = data_content[4:23]

n_dic['卫星钟偏差(s)'] = float((data_content.strip('\n')[23:42]))  # 利用字符串切片功能来进行字符串的修改

n_dic['卫星钟漂移(s/s)'] = float((data_content.strip('\n')[42:61]))

n_dic['卫星钟漂移速度(s/s*s)'] = float((data_content.strip('\n')[61:80]))

if i == 1:

n_dic['IODE'] = float((data_content.strip('\n')[4:23]))

n_dic['C_rs'] = float((data_content.strip('\n')[23:42]))

n_dic['n'] = float((data_content.strip('\n')[42:61]))

n_dic['M0'] = float((data_content.strip('\n')[61:80]))

if i == 2:

n_dic['C_uc'] = float((data_content.strip('\n')[4:23]))

n_dic['e'] = float((data_content.strip('\n')[23:42]))

n_dic['C_us'] = float((data_content.strip('\n')[42:61]))

n_dic['sqrt_A'] = float((data_content.strip('\n')[61:80]))

if i == 3:

n_dic['TEO'] = float((data_content.strip('\n')[4:23]))

n_dic['C_ic'] = float((data_content.strip('\n')[23:42]))

n_dic['OMEGA'] = float((data_content.strip('\n')[42:61]))

n_dic['C_is'] = float((data_content.strip('\n')[61:80]))

if i == 4:

n_dic['I_0'] = float((data_content.strip('\n')[4:23]))

n_dic['C_rc'] = float((data_content.strip('\n')[23:42]))

n_dic['w'] = float((data_content.strip('\n')[42:61]))

n_dic['OMEGA_DOT'] = float((data_content.strip('\n')[61:80]))

if i == 5:

n_dic['IDOT'] = float((data_content.strip('\n')[4:23]))

n_dic['L2_code'] = float((data_content.strip('\n')[23:42]))

n_dic['PS_week_num'] = float((data_content.strip('\n')[42:61]))

if i == 6:

n_dic['卫星精度(m)'] = float((data_content.strip('\n')[4:23]))

n_dic['卫星健康状态'] = float((data_content.strip('\n')[23:42]))

n_dic['TGD'] = float((data_content.strip('\n')[42:61]))

n_dic['IODC'] = float((data_content.strip('\n')[61:80]))

n_dic_list.append(n_dic) # 存储字典

with open('北斗卫星.csv', 'w', newline='') as f:

header = ['数据组数', '卫星PRN号', '历元', '卫星钟偏差(s)', '卫星钟漂移(s/s)', '卫星钟漂移速度(s/s*s)', 'IODE','C_rs', 'n', 'M0', 'C_uc', 'e', 'C_us', 'sqrt_A', 'TEO', 'C_ic', 'OMEGA', 'C_is', 'I_0', 'C_rc', 'w','OMEGA_DOT', 'IDOT', 'L2_code', 'PS_week_num', 'L2_P_code', '卫星精度(m)', '卫星健康状态', 'TGD','IODC','X', 'Y', 'Z']

writer = csv.DictWriter(f, fieldnames=header) # 写入表头

writer.writeheader() # 写入表头

writer.writerows(n_dic_list) # 写入数据

f.close()

prn_x_y_z = [] # 存储卫星位置

with open('北斗卫星.csv', 'rt') as csvfile:

reader = csv.DictReader(csvfile)

for row in reader:

PRN = str(row["卫星PRN号"])

TIME = row["历元"]

year = int(TIME.strip('\n')[2:4])

month = int(TIME.strip('\n')[5:7])

day = int(TIME.strip('\n')[8:10])

hour = int(TIME.strip('\n')[11:13])

minute = int(TIME.strip('\n')[14:16])

second = float(TIME.strip('\n')[17:19])

a_0 = float(row["卫星钟偏差(s)"])

a_1 = float(row["卫星钟漂移(s/s)"])

a_2 = float(row["卫星钟漂移速度(s/s*s)"])

IODE = float(row["IODE"])

C_rs = float(row["C_rs"])

δn = float(row["n"])

M0 = float(row["M0"])

C_uc = float(row["C_uc"])

e = float(row["e"])

C_us = float(row["C_us"])

sqrt_A = float(row["sqrt_A"])

TEO = float(row["TEO"])

C_ic = float(row["C_ic"])

OMEGA = float(row["OMEGA"])

C_is = float(row["C_is"])

I_0 = float(row["I_0"])

C_rc = float(row["C_rc"])

w = float(row["w"])

OMEGA_DOT = float(row["OMEGA_DOT"])

IDOT = float(row["IDOT"])

L2_code = float(row["L2_code"])

PS_week_num = float(row["PS_week_num"])

TGD = float(row["TGD"])

IODC = float(row["IODC"])

t1 = None

# 1.计算卫星运行平均角速度 GM:WGS84下的引力常数 =3.986005e14，a:长半径

GM = 398600500000000

n_0 = m.sqrt(GM) / m.pow(sqrt_A, 3)

n = n_0 + δn

# 2.计算归化时间t_k 计算t时刻的卫星位置  UT：世界时 此处以小时为单位

UT = hour + (minute / 60.0) + (second / 3600)

# GPS时起始时刻1980年1月6日0点   year是两位数 需要转换到四位数

if year >= 80:

if year == 80 and month == 1 and day < 6:

year = year + 2000

else:

year = year + 1900

else:

year = year + 2000

if month <= 2:

year = year - 1

month = month + 12  # 1，2月视为前一年13，14月

# 需要将当前需计算的时刻先转换到儒略日再转换到GPS时间

JD = (365.25 * year) + int(30.6001 * (month + 1)) + day + UT / 24 + 1720981.5

WN = int((JD - 2444244.5) / 7)  # WN:GPS_week number 目标时刻的GPS周

t_oc = ((JD - 2444244.5) - (7.0 * WN)) * 24 * 3600.0 - 14  # t_GPS:目标时刻的GPS秒 减去14秒为BDT

# 对观测时刻t1进行钟差改正,注意：t1应是由接收机接收到的时间

t_k = -14

# 3.平近点角计算M_k = M_0+n*t_k

M_k = M0 + n * t_k  # 实际应该是乘t_k，但是没有接收机的观测时间，所以为了练手设t_k=0

# 4.偏近点角计算 E_k  (迭代计算) E_k = M_k + e*sin(E_k)

E = 0

E1 = 1

count = 0

while abs(E1 - E) > 1e-10:

count = count + 1

E1 = E

E = M_k + e * m.sin(E)

if count > 1e8:

print("计算偏近点角时未收敛！")

break

# 5.计算卫星的真近点角

V_k = m.atan2((m.sqrt(1 - e * e) * m.sin(E)), (m.cos(E) - e));

# 6.计算升交距角 u_0(φ_k), ω:卫星电文给出的近地点角距

u_0 = V_k + w

# 7.摄动改正项 δu、δr、δi :升交距角u、卫星矢径r和轨道倾角i的摄动量

δu = C_uc * m.cos(2 * u_0) + C_us * m.sin(2 * u_0)

δr = C_rc * m.cos(2 * u_0) + C_rs * m.sin(2 * u_0)

δi = C_ic * m.cos(2 * u_0) + C_is * m.sin(2 * u_0)

# 8.计算经过摄动改正的升交距角u_k、卫星矢径r_k和轨道倾角 i_k

u = u_0 + δu

r = m.pow(sqrt_A, 2) * (1 - e * m.cos(E)) + δr

i = I_0 + δi + IDOT * (t_k);  # 实际乘t_k=t-t_oe

# 9.计算卫星在轨道平面坐标系的坐标,卫星在轨道平面直角坐标系（X轴指向升交点）中的坐标为：

x_k = r * m.cos(u)

y_k = r * m.sin(u)

# 10.观测时刻升交点经度Ω_k的计算，升交点经度Ω_k等于观测时刻升交点赤经Ω与格林尼治恒星时GAST之差  Ω_k=Ω_0+(ω_DOT-omega_e)*t_k-omega_e*t_oe

omega_e = 7.292115e-5  # 地球自转角速度

OMEGA_k = OMEGA + (OMEGA_DOT - omega_e) * t_k - omega_e * TEO;  # 星历中给出的Omega即为Omega_o=Omega_t_oe-GAST_w

# 11.计算卫星在地固系中的直角坐标l

X_k = x_k * m.cos(OMEGA_k) - y_k * m.cos(i) * m.sin(OMEGA_k)

Y_k = x_k * m.sin(OMEGA_k) + y_k * m.cos(i) * m.cos(OMEGA_k)

Z_k = y_k * m.sin(i)

# 12.判断卫星是否为GEO卫星，否则不进行极移改正。

if PRN in ['C01', 'C02', 'C03', 'C04', 'C05', 'C59', 'C60', 'C61']:

fi = omega_e * t_k

five = (m.pi / 180 * 5) * (-1)

a = np.asmatrix([X_k, Y_k, Z_k])

a = rz(fi) * rx(five) * a.T

X_k = str(a[0, 0])

Y_k = str(a[1, 0])

Z_k = str(a[2, 0])

if month > 12:  # 恢复历元

year = year + 1

month = month - 12

prn_x_y_z.append(PRN + ",")

prn_x_y_z.append(str(X_k) + ",")

prn_x_y_z.append(str(Y_k) + ",")

prn_x_y_z.append(str(Z_k))

prn_x_y_z.append("\n")

else:

prn_x_y_z.append(PRN + ",")

prn_x_y_z.append(str(X_k) + ",")

prn_x_y_z.append(str(Y_k) + ",")

prn_x_y_z.append(str(Z_k))

prn_x_y_z.append("\n")

return prn_x_y_z

rnx_filename=[]

dir=os.listdir()

for i in range(len(dir)):

line=dir[i]

extension=line.split(".")

if extension[len(extension)-1] in "rnx":

rnx_filename.append(line)

num=1

for i in rnx_filename:

f = open("北斗卫星位置.txt", 'w')

f.writelines(reader_GNSS(i))

print("文件写入成功！")

f.close()

num+=1

print("完成计算的星历名称为：" + str(i))

draw2D.py:

import pandas as pd

import plotly.graph_objects as go

# 读取数据

data = pd.read_csv('北斗卫星位置.txt', header=None, names=['卫星名称', 'x', 'y', 'z'])

# 建立卫星编号与类型的映射关系

def get_satellite_type(satellite):

if satellite in ['C01', 'C02', 'C03', 'C04', 'C05', 'C59', 'C60', 'C61']:

return 'GEO'

elif satellite in ['C06', 'C07', 'C08', 'C09', 'C10', 'C13', 'C16', 'C31', 'C38', 'C39', 'C40']:

return 'IGSO'

else:

return 'MEO'

data['卫星类型'] = data['卫星名称'].apply(get_satellite_type)

# 创建平面散点图

fig = go.Figure()

# 存储所有卫星轨迹的可见性状态，初始都为True

all_visible = []

# 定义不同类型卫星的颜色

type_colors = {'GEO': 'red', 'IGSO': 'orange', 'MEO': 'blue'}

# 存储每个卫星的可见性状态，初始全为 True

visibility_states = []

for satellite, group in data.groupby('卫星名称'):

satellite_type = get_satellite_type(satellite)

fig.add_trace(go.Scatter(

x=group['x'],

y=group['y'],

mode='markers',

name=satellite,

marker=dict(

size=5,

opacity=0.8,

color=type_colors[satellite_type]

),

))

visibility_states.append(True)

# 生成每个卫星单独显示时的可见性列表

buttons = []

# 添加全部显示按钮

buttons.append({

'label': '全部显示',

'method': 'update',

'args': [{'visible': visibility_states},

{'title': '卫星平面散点图'}]

})

# 按卫星类型添加显示按钮

satellite_types = data['卫星类型'].unique()

for sat_type in satellite_types:

visibility = [get_satellite_type(fig.data[i].name) == sat_type for i in range(len(fig.data))]

buttons.append({

'label': sat_type,

'method': 'update',

'args': [{'visible': visibility},

{'title': f'{sat_type} 卫星轨迹'}]

})

for i in range(len(fig.data)):

new_visibility = [False] * len(fig.data)

new_visibility[i] = True

buttons.append({

'label': fig.data[i].name,

'method': 'update',

'args': [{'visible': new_visibility},

{'title': f'{fig.data[i].name} 卫星轨迹'}]

})

# 更新布局，添加下拉菜单

fig.update_layout(

title='卫星平面散点图',

xaxis_title='x坐标',

yaxis_title='y坐标',

plot_bgcolor='white',

paper_bgcolor='white',

updatemenus=[{

'buttons': buttons,

'direction': 'down',

'showactive': True,

'x': 1.1,

'xanchor': 'left',

'y': 1.05,

'yanchor': 'top'

}]

)

fig.show()

#保存为html文件

fig.write_html('卫星平面散点图.html')

draw3D.py:

import pandas as pd

import plotly.graph_objects as go

# 读取数据

data = pd.read_csv('北斗卫星位置.txt', header=None, names=['卫星名称', 'x', 'y', 'z'])

# 建立卫星编号与类型的映射关系

def get_satellite_type(satellite):

if satellite in ['C01', 'C02', 'C03', 'C04', 'C05', 'C59', 'C60', 'C61']:

return 'GEO'

elif satellite in ['C06', 'C07', 'C08', 'C09', 'C10', 'C13', 'C16', 'C31', 'C38', 'C39', 'C40']:

return 'IGSO'

else:

return 'MEO'

data['卫星类型'] = data['卫星名称'].apply(get_satellite_type)

# 创建3D散点图

fig = go.Figure()

# 存储所有卫星轨迹的可见性状态，初始都为True

all_visible = []

# 定义不同类型卫星的颜色

type_colors = {'GEO': 'red', 'IGSO': 'orange', 'MEO': 'blue'}

# 为每个卫星添加轨迹

for satellite, group in data.groupby('卫星名称'):

satellite_type = get_satellite_type(satellite)

fig.add_trace(go.Scatter3d(

x=group['x'],

y=group['y'],

z=group['z'],

mode='markers',

name=satellite,

marker=dict(

size=5,

opacity=0.8,

color=type_colors[satellite_type]

)

))

all_visible.append(True)

# 生成每个卫星单独显示时的可见性列表

buttons = []

# 添加全部显示按钮

buttons.append({

'label': '全部显示',

'method': 'update',

'args': [{'visible': all_visible},

{'title': '卫星立体散点图'}]

})

# 按卫星类型添加显示按钮

satellite_types = data['卫星类型'].unique()

for sat_type in satellite_types:

visibility = [get_satellite_type(fig.data[i].name) == sat_type for i in range(len(fig.data))]

buttons.append({

'label': sat_type,

'method': 'update',

'args': [{'visible': visibility},

{'title': f'{sat_type} 卫星轨迹'}]

})

for i in range(len(fig.data)):

visibility = [False] * len(fig.data)

visibility[i] = True

buttons.append({

'label': fig.data[i].name,

'method': 'update',

'args': [{'visible': visibility},

{'title': f'{fig.data[i].name} 卫星轨迹'}]

})

# 更新布局，添加下拉菜单

fig.update_layout(

scene=dict(

xaxis_title='x坐标',

yaxis_title='y坐标',

zaxis_title='z坐标'

),

title='卫星立体散点图',

updatemenus=[{

'buttons': buttons,

'direction': 'down',

'showactive': True,

'x': 1.1,

'xanchor': 'left',

'y': 1.05,

'yanchor': 'top'

}]

)

fig.show()

#保存为html文件

fig.write_html("卫星立体散点图.html")

sel6-45.py:

# 定义输入文件路径

input_file_path = '北斗卫星位置.txt'

# 定义输出文件路径

output_file_path = '北斗卫星位置_C06-C45.txt'

# 打开输入文件和输出文件

with open(input_file_path, 'r', encoding='utf-8') as input_file, \

open(output_file_path, 'w', encoding='utf-8') as output_file:

# 遍历输入文件的每一行

for line in input_file:

# 去除行首尾的空白字符

line = line.strip()

# 遍历 C06 到 C45 的卫星编号

for i in range(6, 46):

satellite_id = f'C{i:02d}'

# 如果行以当前卫星编号开头

if line.startswith(satellite_id):

# 按逗号分割行数据

parts = line.split(',')

# 提取卫星编号

new_parts = [parts[0]]

# 遍历剩余的数据部分

for part in parts[1:]:

try:

# 将数据转换为浮点数并除以 1000

new_value = float(part) / 1000

# 将结果添加到新的数据部分列表

new_parts.append(str(new_value))

except ValueError:

# 如果转换失败，保持原始数据

new_parts.append(part)

# 用逗号重新连接新的数据部分

new_line = ','.join(new_parts)

# 将新行写入输出文件

output_file.write(new_line + '\n')

break

sp3.py:

# 定义文件路径

file_path = 'WUM0MGXRAP_20250550000_01D_05M_ORB.SP3'

# 用于存储筛选后的数据

filtered_data = []

# 标记是否处于有效的时间行之后

in_valid_time_section = False

# 打开文件并逐行读取

with open(file_path, 'r', encoding='utf-8') as file:

for line in file:

# 检查是否为时间行

if line.startswith('*'):

# 解析时间信息

parts = line.split()

if len(parts) >= 7 and parts[1] == '2025' and parts[2] == '2' and parts[3] == '24' and parts[5] == '0':

# 标记为处于有效时间行之后

in_valid_time_section = True

else:

in_valid_time_section = False

# 检查是否为 PC 开头的卫星数据，并且处于有效时间行之后

elif line.startswith('PC') and in_valid_time_section:

# 删除最后一列数据

parts = line.strip().split()[:-1]

# 去掉首字母 P

if parts[0].startswith('P'):

parts[0] = parts[0][1:]

# 使用逗号分隔各列

new_line = ','.join(parts)

# 将符合条件的数据添加到筛选结果中

filtered_data.append(new_line)

# 仅对第一列进行增序排列

filtered_data.sort(key=lambda x: x.split(',')[0])

# 你也可以将筛选后的数据保存到新文件中

output_file_path = 'sp3_data.txt'

with open(output_file_path, 'w', encoding='utf-8') as output_file:

for data in filtered_data:

output_file.write(data + '\n')

ana.py:

# 读取文件内容

def read_file(file_path):

data = {}

with open(file_path, 'r') as file:

for line in file:

parts = line.strip().split(',')

satellite = parts[0]

if satellite not in data:

data[satellite] = []

data[satellite].append([float(part) for part in parts[1:]])

return data

# 计算差值和百分差

def calculate_differences(data1, data2):

differences = []

percentage_differences = []

for satellite in data1:

if satellite in data2:

for i in range(len(data1[satellite])):

diff = [(data1[satellite][i][j] - data2[satellite][i][j]) * 1000 for j in range(3)]

pct_diff = [(data1[satellite][i][j] - data2[satellite][i][j]) / data2[satellite][i][j] * 100 if

data2[satellite][i][j] != 0 else 0 for j in range(3)]

diff = [round(abs(d), 7) for d in diff]  # 取绝对值

pct_diff = [round(abs(p), 7) for p in pct_diff]  # 取绝对值

differences.append([satellite] + diff)

percentage_differences.append([satellite] + pct_diff)

return differences, percentage_differences

# 将百分差转换为科学计数法并保留7位有效数字

def to_scientific_notation(data):

new_data = []

for row in data:

satellite = row[0]

values = [format(float(num), '.7e') for num in row[1:]]  # 转换全部百分差数据

new_data.append([satellite] + values)

return new_data

# 写入新文件

def write_to_file(differences, percentage_differences, output_path):

with open(output_path, 'w') as file:

for i in range(len(differences)):

diff_str = ','.join([str(d) for d in differences[i]])

pct_diff = to_scientific_notation([percentage_differences[i]])[0]

pct_diff_str = ','.join(pct_diff[1:])  # 去掉卫星编号

file.write(f"{diff_str},{pct_diff_str}\n")

if __name__ == "__main__":

file1_path = '北斗卫星位置_C06-C45.txt'

file2_path = 'sp3_data.txt'

output_path = 'ana.txt'

data1 = read_file(file1_path)

data2 = read_file(file2_path)

differences, percentage_differences = calculate_differences(data1, data2)

write_to_file(differences, percentage_differences, output_path)

config.py:

def calculate_column_average(file_path):

data = []

with open(file_path, 'r') as file:

for line in file:

line = line.strip()

if line:

values = list(map(float, line.split(',')[1:]))

data.append(values)

num_columns = len(data[0])

column_sums = [0] * num_columns

for row in data:

for i in range(num_columns):

column_sums[i] += abs(row[i])

column_averages = [sum_value / len(data) for sum_value in column_sums]

return column_averages

def write_to_config(averages, output_path):

with open(output_path, 'w') as file:

headers = ["x坐标差值平均值", "y坐标差值平均值", "z坐标差值平均值",

"x坐标百分差平均值", "y坐标百分差平均值", "z坐标百分差平均值"]

for header, average in zip(headers, averages):

file.write(f"{header}: {average}\n")

if __name__ == "__main__":

input_file = "ana.txt"

output_file = "config.txt"

averages = calculate_column_average(input_file)

write_to_config(averages, output_file)

plot.py:

import pandas as pd

import matplotlib.pyplot as plt

import numpy as np

def plot_boxplots(file_path):

# 读取数据到DataFrame

df = pd.read_csv(file_path, header=None, names=['id', 'x', 'y', 'z', 'x_percent', 'y_percent', 'z_percent'])

# 提取差值数据列

diff_columns = ['x', 'y', 'z']

diff_data = [df[col] for col in diff_columns]

# 提取百分差数据列

percent_diff_columns = ['x_percent', 'y_percent', 'z_percent']

percent_diff_data = [df[col] for col in percent_diff_columns]

# 计算差值数据列的绝对值平均值

diff_abs_mean = [np.mean(np.abs(df[col])) for col in diff_columns]

# 计算百分差数据列的绝对值平均值

percent_diff_abs_mean = [np.mean(np.abs(df[col])) for col in percent_diff_columns]

# 绘制差值数据的箱线图

plt.figure(figsize=(10, 6))

positions_diff = np.arange(len(diff_columns))

bp_diff = plt.boxplot(diff_data, positions=positions_diff, widths=0.5)

plt.xticks(positions_diff, diff_columns)

plt.xlabel('Coordinate Difference Columns')

plt.ylabel('Values')

plt.title('Boxplots of Coordinate Differences')

for i, box in enumerate(bp_diff['boxes']):

y = box.get_ydata()[1]

plt.text(positions_diff[i], y, f'{diff_abs_mean[i]:.4f}', ha='center', va='bottom')

plt.grid(True, axis='y', linestyle='--', alpha=0.7)

plt.show()

if __name__ == "__main__":

input_file = "ana.txt"

plot_boxplots(input_file)

#保存图片

plt.savefig("boxplot.png")

plot1.py:

import pandas as pd

import matplotlib.pyplot as plt

import numpy as np

def plot_scatter_with_mean(file_path):

# 读取数据到DataFrame

df = pd.read_csv(file_path, header=None, names=['id', 'x', 'y', 'z', 'x_percent', 'y_percent', 'z_percent'])

# 计算x、y、z差值的绝对值平均值

x_mean = np.mean(np.abs(df['x']))

y_mean = np.mean(np.abs(df['y']))

z_mean = np.mean(np.abs(df['z']))

# 创建子图

fig, axes = plt.subplots(1, 3, figsize=(15, 5))

# 绘制x差值的散点图，纵坐标使用绝对值

axes[0].scatter(range(len(df['x'])), np.abs(df['x']))

axes[0].axhline(y=x_mean, color='r', linestyle='--', label=f'Abs Mean: {x_mean:.4f}')

axes[0].set_title('X Difference Scatter Plot')

axes[0].set_xlabel('Data Point Index')

axes[0].set_ylabel('Absolute X Difference Value')

axes[0].legend()

# 绘制y差值的散点图，纵坐标使用绝对值

axes[1].scatter(range(len(df['y'])), np.abs(df['y']))

axes[1].axhline(y=y_mean, color='r', linestyle='--', label=f'Abs Mean: {y_mean:.4f}')

axes[1].set_title('Y Difference Scatter Plot')

axes[1].set_xlabel('Data Point Index')

axes[1].set_ylabel('Absolute Y Difference Value')

axes[1].legend()

# 绘制z差值的散点图，纵坐标使用绝对值

axes[2].scatter(range(len(df['z'])), np.abs(df['z']))

axes[2].axhline(y=z_mean, color='r', linestyle='--', label=f'Abs Mean: {z_mean:.4f}')

axes[2].set_title('Z Difference Scatter Plot')

axes[2].set_xlabel('Data Point Index')

axes[2].set_ylabel('Absolute Z Difference Value')

axes[2].legend()

plt.tight_layout()

plt.show()

if __name__ == "__main__":

input_file = "ana.txt"

plot_scatter_with_mean(input_file)

# 保存到文件

plt.savefig("point.png")

- 结果

1.卫星坐标位置计算结果 北斗卫星位置.txt：

2.卫星坐标图绘制结果（红色GEO，橙色IGSO，蓝色MEO）：

3.误差及百分差计算结果（单位：m） ana.txt：

4.整体计算精度评定（单位：m） config.txt：

绘制各卫星x轴, y轴, z轴误差的箱线图。从图中可以看出：各轴数据的最大误差均小于2m,最小误差接近0m，三轴平均误差分别为0.7902m, 0.7573m, 0.6319m,存在一定的异常值。

绘制各卫星x轴, y轴, z轴误差的散点分布图。从图中可以看出：y轴的计算误差整体不存在极端值，且编号在C06-C15的数据计算误差较大，其余编号的卫星的误差分布比较理想。

- 总结

本次课后作业利用卫星广播星历计算出了卫星的位置计算值，并与真值进行了误差计算和进度评估。首先，从指定网站下载广播星历文件，经数据预处理，提取北斗卫星数据并解析存入字典，再依据一系列公式计算卫星坐标位置，输出结果文件并绘制 2D、3D 图。同时，对精密星历数据预处理，与计算值对比计算误差和百分差，绘制误差散点图和箱线图评估精度。

从结果来看，成功计算出卫星坐标位置，绘制出卫星坐标图，清晰展示不同类型卫星分布。误差及百分差计算结果表明，不同卫星在各坐标方向上存在一定误差。整体计算精度评定显示，x、y、z 坐标差值平均值分别为 0.7901932350694444m、0.7572539730324068m、0.6318650188657409m，百分差平均值也处于10-5数量级。此次作业完整实现了从数据处理到结果分析的流程。

通过这次作业，在知识层面，我深入理解了北斗卫星广播星历的构成和应用，以及卫星轨道计算的原理和方法，对卫星导航定位系统的运行机制有了更清晰的认知。在计算卫星位置时，部分公式理解困难，迭代计算偏近点角时还遇到未收敛的问题，花费不少时间排查解决。这次作业也让我认识到卫星导航定位领域的严谨性和复杂性，每个数据、每个步骤都至关重要。